ECA1414 – EMBEDDED SYSTEMS – SLOT – D – CO2 – AT3                             MARKS: 100

| Criteria | Weightage | Marks |
| --- | --- | --- |
| Problem Understanding | 20% (20) |  |
| Algorithm | 25%(25) |  |
| Code Implementation | 20%(20) |  |
| Competitive Performance | 20%(20) |  |

Code of Conduct:

I ________GUNTHA BHARATHI / 192371079____________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: GUNTHA BHARATHI

- Requirements Analysis & Conceptual Understanding (20%)

  - Problem Statement

Embedded and industrial systems frequently decompose into independently-clocked producer and consumer tasks — a sensor ISR generating samples faster than a communication task can transmit them, or a fast peripheral feeding a slower processing pipeline. A FIFO (First-In-First-Out) channel decouples the two, but introduces queueing latency and, once the channel saturates, backpressure that throttles the producer. This task models exactly that scenario in SystemC: a Producer module publishes timestamped frames into an sc_fifo channel every 20 ms; a Consumer module reads and services frames every 35 ms; the model logs each transaction and computes the resulting latency statistics.

  - Why a FIFO Channel (Conceptual Basis)

    - A FIFO channel provides ordered, lossless hand-off between two processes running at different, independent rates — exactly the relationship between an ISR-driven producer and a slower consumer task in embedded firmware.

    - SystemC's sc_fifo<T> is a built-in primitive channel offering blocking write() when full and blocking read() when empty, which naturally models hardware FIFO/ring-buffer peripherals (e.g., UART FIFOs, DMA queues) without hand-written synchronization logic.

    - Bounding the FIFO's capacity is deliberate: an unbounded queue would hide the ratemismatch; a bounded queue exposes it as measurable backpressure and rising latency, which is what real embedded buffers experience under load.

  - Key SystemC / Embedded C Concepts Applied

| Concept | Application in this Model |
| --- | --- |
| sc_fifo<T> primitive channel | Bounded 4-entry queue carrying Frame_t structures between modules |
| Blocking write()/read() | Producer stalls when the channel is full; Consumer stalls when empty — models real hardware FIFO backpressure |
| SC_THREAD + wait(N, SC_MS) | Periodic task behaviour: Producer every 20 ms, Consumer every 35 ms |
| sc_time_stamp() / sc_time arithmetic | Timestamping each frame at production and consumption to compute latency |
| Fixed-format frame struct (id, timestamp) | Embedded C style POD (plain-old-data) message format, portable to a real bus frame |
| num_free() / num_available() | Runtime introspection of channel occupancy, used to observe saturation |

  - Functional Requirements

    - Model two independent, periodically-triggered modules (Producer, Consumer) connected only through a bounded FIFO channel.

    - Timestamp every frame at the moment it is written to the channel.

    - Compute per-frame latency as (time of read) − (time of write) at the consumer.

    - Track channel occupancy (num_free / num_available) to identify saturation and backpressure.

    - Report aggregate latency statistics — minimum, maximum, and average — at the end of the simulation run.

  - Real-Time / Non-Functional Constraints

| Constraint | Type | Expectation |
| --- | --- | --- |
| Producer period | Periodic, deterministic | 20 ms ± 0 (ideal sc_clockderived tick) |
| Consumer period | Periodic, deterministic | 35 ms ± 0 |
| FIFO channel capacity | Design parameter | 4 frames (bounded, nonblocking-full only up to this limit) |
| Steady-state latency bound | Derived (soft realtime) | Should converge to a stable ceiling once the channel saturates, not grow unbounded |

- System Design & Simulation Model Development (25%)

  - Architecture — Block Diagram

The model consists of exactly three elements: a Producer sc_module, an sc_fifo<Frame> channel of fixed capacity, and a Consumer sc_module. This mirrors the shared-bus abstraction used elsewhere in embedded firmware (e.g., the sensor-to-HMI frame path in a monitoring node), reduced here to its essential communication primitive so the latency behaviour can be isolated and measured cleanly.

SystemC FIFO CHANNEL COMMUNICATION MODEL

+---------------+        sc_fifo<Frame>        +---------------+

|   Producer    |   write()         read()     |   Consumer    |

|  sc_module    | ----------> [ FIFO ] -------->|  sc_module    |

¦ ¦ capacity = 4 frames ¦ ¦ ¦ SC_THREAD ¦ blocks on FULL ¦ SC_THREAD ¦ ¦ period 20 ms ¦ blocks on EMPTY ¦ period 35 ms¦ +---------------+ +---------------+ ¦ ¦ v v Frame{id, t_produced} latency = t_read - t_produced (logged + aggregated)

  - Frame (Message) Format

Each transaction carries a minimal, fixed-layout structure — consistent with Embedded C's preference for compact, register-friendly data types over dynamically-sized messages.

struct Frame {     unsigned id;          // monotonically increasing sequence number     sc_time  t_produced;  // simulation timestamp at write() };

  - State / Interaction Model

Producer thread Consumer thread ---------------- ---------------- loop: loop: wait(20 ms) wait(35 ms) build Frame(id, now) f = fifo.read() <-- blocks if empty fifo.write(f) <-- blocks if full latency = now - f.t_produced id++ record(latency) Net effect once FIFO saturates: producer's effective write rate is throttled down to the consumer's 35 ms cadence (observed directly in Section 4).

  - SystemC Modelling Approach

| SystemC Construct | Role in this Model |
| --- | --- |
| sc_fifo<Frame> fifo_ch(4) | The communication channel under test — capacity fixed at 4 frames |
| sc_fifo_out<Frame> / sc_fifo_in<Frame> | Producer/Consumer module ports bound to the channel |
| SC_THREAD(produce) / SC_THREAD(consume) | Independent periodic processes, each with wait()-based timing |
| SystemC Construct | Role in this Model |
| out->num_free() / in>num_available() | Live channel-occupancy introspection printed per transaction |
| sc_time_stamp(), sc_time subtraction | Latency = read time − write time, computed per frame |
| sc_start(2, SC_SEC) | Drives the simulation for 2 simulated seconds of traffic |

. Implementation & Tool Usage (20%)

3.1 Tools Used

| Tool | Purpose |  |
| --- | --- | --- |
| SystemC 2.3.4 (Accellera) class library, C++17 | Event-driven simulation of the Producer / FIFO / Consumer model |  |
| g++ 13.3 (Ubuntu 24.04 toolchain) | Compilation: g++ -std=c++17 -o fifo_latency fifo_latency.cpp lsystemc -lm |  |
| Online C++ compiler (portable variant) | A dependency-free C re-implementation of the same logic, for environments without SystemC installed |  |
| GitHub | Version control and submission of source and this report |  |
| // fifo_latency.cpp -- SystemC model of FIFO-channel communication between a // Producer and a Consumer module, with per-frame end-to-end latency logging. #include <systemc.h> struct Frame { unsigned id; sc_time t_produced; }; inline std::ostream& operator<<(std::ostream& os, const Frame& f) { os << "Frame{id=" << f.id << ",t=" << f.t_produced << "}"; return os; } SC_MODULE(Producer) { sc_fifo_out<Frame> out; unsigned next_id; |  |

void produce() { while (true) { wait(20, SC_MS); // producer period Frame f{next_id++, sc_time_stamp()}; out.write(f); // blocks if FIFO full cout << "[" << sc_time_stamp() << "] PRODUCER wrote frame #" << f.id << " (fifo free=" << out->num_free() << ")" << endl; } } SC_CTOR(Producer) : next_id(0) { SC_THREAD(produce); } }; SC_MODULE(Consumer) { sc_fifo_in<Frame> in; unsigned count; double total_latency_ms, min_latency_ms, max_latency_ms; void consume() { while (true) { wait(35, SC_MS); // consumer period (slower -> backpressure) Frame f = in.read(); // blocks if FIFO empty double latency_ms = (sc_time_stamp() - f.t_produced).to_seconds() * 1000.0; total_latency_ms += latency_ms; if (count == 0 ¦¦ latency_ms < min_latency_ms) min_latency_ms = latency_ms; if (latency_ms > max_latency_ms) max_latency_ms = latency_ms; count++; cout << "[" << sc_time_stamp() << "] CONSUMER read frame #" << f.id << " latency=" << latency_ms << " ms" << " (fifo num_available=" << in->num_available() << ")" << endl; } } SC_CTOR(Consumer) : count(0), total_latency_ms(0.0), min_latency_ms(0.0), max_latency_ms(0.0) { SC_THREAD(consume); } }; int sc_main(int argc, char* argv[]) { sc_fifo<Frame> fifo_ch(4); // FIFO channel, capacity = 4 frames Producer u_prod("u_prod"); u_prod.out(fifo_ch); Consumer u_cons("u_cons"); u_cons.in(fifo_ch); cout << "===== SystemC FIFO Channel Communication + Latency Analysis =====" << endl; cout << "FIFO capacity=4 ¦ Producer period=20ms ¦ Consumer period=35ms" << endl << endl;

sc_start(2, SC_SEC); cout << endl << "===== Latency Analysis Summary =====" << endl; cout << "Frames produced : " << u_prod.next_id << endl; cout << "Frames consumed : " << u_cons.count << endl; cout << "Min latency : " << u_cons.min_latency_ms << " ms" << endl; cout << "Max latency : " << u_cons.max_latency_ms << " ms" << endl; cout << "Average latency : " << (u_cons.count ? u_cons.total_latency_ms / u_cons.count : 0.0) << " ms" << endl; return 0; }

  - Build Command

$ g++ -std=c++17 -o fifo_latency fifo_latency.cpp -lsystemc -lm $ ./fifo_latency

  - Portable Variant for Generic Online Compilers

Because most general-purpose online compilers do not have the SystemC headers/library installed, the same producer–FIFO–consumer behaviour was re-expressed as a dependency-free C program using a manually-implemented circular-buffer channel and a millisecond time-step loop in place of sc_fifo/SC_THREAD/wait(). This lets the model and its latency analysis be reproduced on any standard C/C++ online compiler without additional setup, and produces the same qualitative result (latency rising to a steady ceiling once the buffer saturates).

| SystemC Construct | Portable C Equivalent |
| --- | --- |
| sc_fifo<Frame>(4) | Fixed-size circular buffer, head/tail/count indices |
| out.write() / in.read() (blocking) | fifo_write()/fifo_read() returning false when full/empty; caller retries on the next time-step |
| wait(N, SC_MS) | if (t_ms % N == 0) { ... } inside a for-loop stepping t_ms by 1 ms |
| sc_time_stamp() | The t_ms loop variable |

- Testing, Validation & Result Analysis (20%) 4.1 Test Plan

| ID | Test Case | Expected Result | Observed |
| --- | --- | --- | --- |
| TC01 | Channel starts empty; first frame written and read | Minimal latency (≈ one consumer period offset) | Frame #0: latency = 15 ms — as expected |
| TC02 | Producer (20 ms) faster than Consumer (35 ms) over a sustained run | FIFO occupancy rises; latency increases per frame until saturation | Confirmed: latency climbs from 15 ms to a steady 155 ms |
| TC03 | FIFO reaches capacity (4 frames) | Producer's next write() blocks until Consumer frees a slot (num_free reported as 0) | Confirmed: fifo free=0 sustained after ≈140 ms; producer write cadence throttles to 35 ms |
| TC04 | Long-run steady state | Latency converges to a stable ceiling rather than growing without bound | Confirmed: latency plateaus at 155 ms from frame #10 onward |
| TC05 | End-of-run statistics | Aggregate min/max/average latency computed correctly across all consumed frames | Min = 15 ms, Max = 155 ms, Average = 142.28 ms over 57 consumed frames |

4.2 Simulation Output (Genuine Run — g++ 13.3 / SystemC 2.3.4)

The excerpt below is the unedited startup and transient-phase output of the compiled and executed model (full run: 2 simulated seconds).

$ g++ -std=c++17 -o fifo_latency fifo_latency.cpp -lsystemc -lm $ ./fifo_latency SystemC 2.3.4-Accellera --- Apr 22 2024 Copyright (c) 1996-2022 by all Contributors, ALL RIGHTS RESERVED ===== SystemC FIFO Channel Communication + Latency Analysis ===== FIFO capacity=4 ¦ Producer period=20ms ¦ Consumer period=35ms [20 ms] PRODUCER wrote frame #0 (fifo free=3) [35 ms] CONSUMER read frame #0 latency=15 ms (fifo num_available=0)

[40 ms] PRODUCER wrote  frame #1  (fifo free=3)

[60 ms] PRODUCER wrote  frame #2  (fifo free=2)

[70 ms] CONSUMER read   frame #1  latency=30 ms  (fifo num_available=1)

[80 ms] PRODUCER wrote  frame #3  (fifo free=2)

[100 ms] PRODUCER wrote  frame #4  (fifo free=1)

[105 ms] CONSUMER read   frame #2  latency=45 ms  (fifo num_available=2)

[120 ms] PRODUCER wrote  frame #5  (fifo free=1)

[140 ms] CONSUMER read   frame #3  latency=60 ms  (fifo num_available=2)

[140 ms] PRODUCER wrote  frame #6  (fifo free=0)

[160 ms] PRODUCER wrote  frame #7  (fifo free=0)

[175 ms] CONSUMER read   frame #4  latency=75 ms  (fifo num_available=3)   ...  (channel saturates; producer write cadence throttles to 35 ms)  ...

Steady-state excerpt (from ≈350 ms onward), showing latency plateaued at the channel's saturation ceiling:

[350 ms] CONSUMER read   frame #9   latency=150 ms  (fifo num_available=3)

[350 ms] PRODUCER wrote  frame #13  (fifo free=0)

[385 ms] CONSUMER read   frame #10  latency=155 ms  (fifo num_available=3)

[385 ms] PRODUCER wrote  frame #14  (fifo free=0)

[420 ms] CONSUMER read   frame #11  latency=155 ms  (fifo num_available=3)

[420 ms] PRODUCER wrote  frame #15  (fifo free=0)

...  (latency holds steady at 155 ms for the remainder of the run)  ...

Final summary block printed at the end of the 2-second run:

===== Latency Analysis Summary =====

Frames produced      : 61

Frames consumed      : 57

Min latency          : 15 ms

Max latency          : 155 ms

Average latency      : 142.281 ms

4.3 Result Analysis

# Latency Behaviour

- Transient phase (0–140 ms): the FIFO is not yet full, so each frame's latency simply reflects how far into the Consumer's 35 ms cycle it happened to arrive — latency rises roughly linearly (15, 30, 45, 60, 75 ms…) as the offset between the Producer's faster 20 ms cadence and the Consumer's cycle accumulates.

- Saturation point (≈140 ms): fifo free reaches 0 for the first time — the channel is now always full between consumer reads, and the Producer's next write() call blocks until a slot is freed.

- Steady state (≈385 ms onward): latency stabilizes at a constant 155 ms. Once blocked, the

Producer's effective write rate is pulled down to match the Consumer's 35 ms period, so

every subsequent frame waits almost exactly one full FIFO-depth's worth of consumer cycles before being read — a textbook demonstration of blocking-channel backpressure.

# Throughput vs. Rate Mismatch

- 61 frames were produced in 2 s (a nominal rate of 1 per 20 ms = 50/s), but only 57 were consumed by the same instant, confirming the Consumer's fixed 35 ms period is the binding constraint on end-to-end throughput, not the Producer.

- Average latency (142.28 ms) sits close to the steady-state ceiling (155 ms) because the short low-latency transient phase is a small fraction of the 2-second run.

# Design Implication

For a real embedded deployment, this result would indicate that either the consumer task must be sped up (or its work-per-cycle reduced), the FIFO capacity increased to absorb longer bursts without immediate blocking, or the producer's rate throttled/paced to match the consumer — all three are direct, quantified design levers exposed by this latency analysis before any hardware is built.

- Documentation (15%)

  - Repository Structure

fifo-latency-systemc/ ¦-- systemc_model/ ¦ `-- fifo_latency.cpp // sc_fifo Producer/Consumer model (Section 3.2) ¦-- portable_c/ ¦ `-- fifo_latency_sim.c // dependency-free C version for online compilers ¦-- results/ ¦ `-- fifo_run_output.txt // full simulation transcript `-- docs/ `-- FIFO_Latency_Report.docx

  - Reproduction Instructions (for the README)

- Install SystemC (e.g. apt install libsystemc-dev on Ubuntu, or any SystemC 2.3.x distribution).

- Build: g++ -std=c++17 -o fifo_latency fifo_latency.cpp -lsystemc -lm

- Run: ./fifo_latency — output streams to stdout and matches Section 4.2 exactly.

- For environments without SystemC, compile portable_c/fifo_latency_sim.c with any standard C compiler (or paste into an online compiler) to reproduce the same qualitative latency behaviour.

5.3 Conclusion

This task constructed a minimal but complete SystemC model of FIFO-channel communication between two independently-clocked modules, and used it to analyze the latency and backpressure that arise from a producer/consumer rate mismatch — a pattern directly applicable to sensor-tobus and bus-to-HMI links in embedded and Industry 5.0 systems. The genuine simulation run confirms the expected transient-then-steady-state latency profile, quantifies the steady-state latency ceiling at 155 ms for the chosen periods and channel depth, and demonstrates the blocking write()/read() semantics that make sc_fifo an accurate stand-in for real hardware FIFO peripherals, satisfying the structured design, real-time constraint handling, and SystemC-based modelling expectations of CO2 (BL5).

.2 SystemC Implementation — Producer, Consumer, and Channel Wiring

The model below was written, compiled, and executed exactly as shown; results are reported directly from this run in Section 4.